RT-5D DMR Transceiver

Data Structure Reference

Complete byte-level documentation of all data blocks exchanged during programming

Derived from CPS source code (JJCC-888DMR_CPS) — February 2026

# Document Conventions

This document describes every byte of every data block sent between the CPS host software and the RT-5D radio over its programming serial link. Blocks are listed in the order they appear in the protocol sequence.

| Convention | Meaning |
| --- | --- |
| 0xFF | Byte value 255 decimal. Used throughout as a padding/fill byte and as the empty-entry sentinel in list structures. |
| 0x00 | Byte value 0 decimal. |
| little-endian (LE) | Multi-byte integers stored least-significant byte first. |
| big-endian (BE) | Multi-byte integers stored most-significant byte first. |
| nibble (low) | Bits 3:0 of a byte. Bit-mask: value & 0x0F. |
| nibble (high) | Bits 7:4 of a byte. Bit-mask: (value >> 4) & 0x0F. |
| GB2312 | Chinese national standard character encoding. ASCII-compatible for codepoints ≤ 0x7F; Chinese characters use 2 bytes ≥ 0x80. |
| Default value | The value the CPS pre-populates when no radio data has been loaded. |

All sizes are in bytes. All offsets are relative to the start of the block payload (i.e., byte 0 of the payload data following the 6-byte frame header).

# 1. Handshake Block

| Property | Value |
| --- | --- |
| Command (Read/Write) | 0x02 (CmdHandshake) |
| Packet count | 1 |
| Payload size | 15 bytes |
| Direction | Host → Radio (request), Radio → Host (response same cmd) |

The handshake payload is a fixed 15-byte ASCII string that identifies the CPS to the radio and places the radio into programming mode. The radio echoes back a valid framed response using the same command byte. The CPS does not validate the content of the radio's reply body, only that a correctly-framed packet is received.

| Offset (bytes) | Size | Field Name | Encoded Values / Notes |
| --- | --- | --- | --- |
| 0 – 14 | 15 | Handshake string | ASCII: PROGRAMJC8810DU (fixed, hardcoded) |

Hex representation of the handshake payload:

50 52 4F 47 52 41 4D 4A 43 38 38 31 30 44 55

# 2. Password Block

| Property | Value |
| --- | --- |
| Command | 0x05 (CmdCheckPwd) |
| Packet count | 1 |
| Payload size | 6 bytes |
| Direction | Host → Radio |

Sends the terminal unlock password to the radio. The CPS always sends 6 bytes of 0xFF, representing the default blank/no-password state. The radio validates this against its stored password. A NAK (0xEE) response command byte indicates authentication failure.

| Offset (bytes) | Size | Field Name | Encoded Values / Notes |
| --- | --- | --- | --- |
| 0 – 5 | 6 | Password bytes | Fixed: FF FF FF FF FF FF (default blank password) |

# 3. Version Block

| Property | Value |
| --- | --- |
| Command | 0x46 (CmdGetVersion) |
| Packet count | 1 |
| Payload size | 128 bytes |
| Direction | Host → Radio (request with 128 zero bytes); Radio → Host (response with version data) |

The host sends 128 zero bytes as a placeholder read request. The radio responds with 128 bytes of version and identity information. The CPS acknowledges receipt but does not parse or act on the content of the version response — it serves only as a handshake confirmation that the radio model is compatible.

| Offset (bytes) | Size | Field Name | Encoded Values / Notes |
| --- | --- | --- | --- |
| 0 – 127 | 128 | Version data | Content defined by radio firmware. CPS sends all 0x00 as request; radio responds with version string and firmware build information. |

# 4. DTMF Block

| Property | Value |
| --- | --- |
| Read command | 0x16 (CmdReadDtmf) |
| Write command | 0x36 (CmdWriteDtmf) |
| Packet count | 1 |
| Payload size | 272 bytes total |
| Sequence number | 0 (single packet, always 0) |

The DTMF block carries the radio’s DTMF (Dual-Tone Multi-Frequency) configuration in two contiguous sub-regions: a 32-byte global settings area followed by a 240-byte code group list.

## 4.1 DTMF Global Settings (bytes 0–31)

| Offset (bytes) | Size | Field Name | Encoded Values / Notes |
| --- | --- | --- | --- |
| 0 – 4 | 5 | DtmfCurId | Current DTMF ID digits. Each byte encodes one digit as an index into the alphabet "0123456789ABCD*#" (values 0–15). Unused digit positions are 0xFF. Default: "12345" → [01 02 03 04 05]. |
| 5 | 1 | (reserved) | 0xFF padding. |
| 6 | 1 | DtmfPttId | Low nibble only (& 0x0F), modulo 4. See value table below. |
| 7 | 1 | DtmfDuration | Low nibble only, modulo 5. See value table below. |
| 8 | 1 | DtmfInterval | Low nibble only, modulo 5. See value table below. |
| 9 – 31 | 23 | (reserved) | 0xFF padding. |

### DtmfPttId values (byte 6, low nibble)

| Value (decimal) | Meaning |
| --- | --- |
| 0 | OFF — no PTT ID transmitted |
| 1 | BOT (Beginning of Transmission) — send ID when PTT pressed |
| 2 | EOT (End of Transmission) — send ID when PTT released |
| 3 | BOTH — send ID at start and end of transmission |

### DtmfDuration values (byte 7, low nibble)

| Value (decimal) | Meaning |
| --- | --- |
| 0 | 50 ms |
| 1 | 100 ms (default) |
| 2 | 150 ms |
| 3 | 200 ms |
| 4 | 250 ms |

### DtmfInterval values (byte 8, low nibble)

| Value (decimal) | Meaning |
| --- | --- |
| 0 | 50 ms |
| 1 | 100 ms (default) |
| 2 | 150 ms |
| 3 | 200 ms |
| 4 | 250 ms |

## 4.2 DTMF Code Group List (bytes 32–271)

15 code groups, each occupying 16 bytes, packed contiguously. Group N starts at offset 32 + (N × 16). An entry whose first byte is 0xFF is treated as empty.

| Offset within entry | Size | Field | Encoding |
| --- | --- | --- | --- |
| 0 – 5 | 6 | DtmfCode digits | Each byte = digit index 0–15 in alphabet "0123456789ABCD*#". Fewer than 6 digits: remaining bytes are 0xFF. First byte 0xFF → entry is empty. |
| 6 – 15 | 10 | (reserved padding) | 0xFF |

Character alphabet: index 0='0', 1='1', ..., 9='9', 10='A', 11='B', 12='C', 13='D', 14='*', 15='#'.

# 5. Encryption Key Block

| Property | Value |
| --- | --- |
| Read command | 0x15 (CmdReadEncKey) |
| Write command | 0x35 (CmdWriteEncKey) |
| Packet count | 1 |
| Payload size | 264 bytes |
| Capacity | 8 key entries |

Carries up to 8 encryption key definitions. Each entry is 33 bytes. Entry N starts at offset N × 33. An entry whose bytes 0 and 1 are both 0xFF is treated as empty/unused.

## 5.1 Key Entry Structure (33 bytes per entry)

| Offset (bytes) | Size | Field Name | Encoded Values / Notes |
| --- | --- | --- | --- |
| 0 | 1 | EncAlgo | Low nibble (& 0x0F), modulo 3. Encryption algorithm selector. See table below. |
| 1 – 32 | 32 | EncKey digits | Each byte = hex digit index 0–15 in alphabet "0123456789ABCDEF". Bytes 0xFF indicate unused digit positions. Number of significant bytes depends on algorithm. |

### EncAlgo values (byte 0, low nibble)

| Value | Algorithm | Key Length (bytes) | Key Length (hex chars) | Notes |
| --- | --- | --- | --- | --- |
| 0 | ARC4 (RC4 compatible) | 5 | 10 | Proprietary basic scrambling |
| 1 | AES-128 | 16 | 32 | Standard AES 128-bit |
| 2 | AES-256 | 32 | 64 | Standard AES 256-bit |

When writing, the CPS pads the key string with '0' characters on the right to reach the required length for the selected algorithm, then truncates to exactly that length. The bytes beyond the required length in the 32-byte field are set to 0xFF.

# 6. Address Book Block

| Property | Value |
| --- | --- |
| Read command | 0x13 (CmdReadAddrBook) |
| Write command | 0x33 (CmdWriteAddrBook) |
| Packet count | 80 |
| Payload size per packet | 800 bytes |
| Total payload | 64,000 bytes |
| Contacts per packet | 50 |
| Maximum total contacts | 4,000 |
| Sequence number | 0–79 (increments per packet) |

The address book (digital contact list / DMR ID book) is split into 80 consecutive packets. Packet N carries the 50 contacts at positions N×50 through N×50+49 in the full contact list. Each contact record is 16 bytes. A contact whose byte 0, byte 1, or byte 5 is 0xFF is ignored. The CPS also ignores All-Call entries during import.

## 6.1 Contact Record Structure (16 bytes)

| Offset (bytes) | Size | Field Name | Encoded Values / Notes |
| --- | --- | --- | --- |
| 0 | 1 | CallType | Low nibble (& 0x0F), modulo 3. Call type selector. See table below. |
| 1 | 1 | (zero byte) | Must be 0x00 for a valid entry. If 0xFF, the record is treated as empty. |
| 2 | 1 | CallId MSB | Most significant byte of the 24-bit DMR ID (big-endian). Byte 1 of the ID integer. |
| 3 | 1 | CallId mid | Middle byte of the 24-bit DMR ID. |
| 4 | 1 | CallId LSB | Least significant byte of the 24-bit DMR ID. |
| 5 – 14 | 10 | ContactName | GB2312 encoded string. Null (0x00) or 0xFF terminated. If byte 5 is 0xFF, the entry is treated as empty. |
| 15 | 1 | (padding) | 0xFF |

### CallType values (byte 0, low nibble)

| Value (decimal) | Meaning |
| --- | --- |
| 0 | Group Call |
| 1 | Private Call (Individual) |
| 2 | All Call |

### CallId encoding

The DMR ID is a 24-bit unsigned integer stored in bytes 1–4 of the record:

- Byte 1 = always 0x00 (the MSByte of the 32-bit representation is zero, confirming 24-bit ID range)

- Byte 2 = (CallId >> 16) & 0xFF

- Byte 3 = (CallId >> 8) & 0xFF

- Byte 4 = CallId & 0xFF

Valid range: 1 to 16,777,215 (0x000001 to 0xFFFFFF). The CPS enforces this range on both read and write.

Note: The default address book contains one entry: CallType=2 (All Call), CallId=16,777,215, ContactName="Call.All". This entry is protected — it is not exported to the radio and is re-added automatically if deleted.

# 7. Receive Group List Block

| Property | Value |
| --- | --- |
| Read command | 0x14 (CmdReadRxGroup) |
| Write command | 0x34 (CmdWriteRxGroup) |
| Packet count | 4 |
| Payload size per packet | 1,024 bytes |
| Total payload | 4,096 bytes |
| Groups per packet | 8 |
| Maximum total groups | 32 |
| Maximum members per group | 32 |
| Sequence number | 0–3 (increments per packet) |

The Rx Group List defines sets of DMR contacts (private call IDs) that a channel can listen to selectively. Each packet carries 8 group entries. Group N in a packet starts at byte (N × 128) within the packet payload. An entry whose byte 96 (the first byte of the name field) is 0xFF is treated as empty.

## 7.1 Group Entry Structure (128 bytes per entry)

| Offset (bytes) | Size | Field Name | Encoded Values / Notes |
| --- | --- | --- | --- |
| 0 – 95 | 96 | Member ID list | Up to 32 member DMR IDs, each 3 bytes (24-bit, big-endian). Unused member slots are three 0x00 bytes. First slot with 0x00 0x00 0x00 marks end of list. |
| 96 – 107 | 12 | RxGroupName | GB2312 encoded string. Null (0x00) or 0xFF terminated. First byte 0xFF → entry is empty. |
| 108 – 127 | 20 | (padding) | 0xFF |

## 7.2 Member ID Encoding (3 bytes per member)

Each member occupies bytes [i×3], [i×3+1], [i×3+2] within the 96-byte member block:

- Byte 0 of member = (CallId >> 16) & 0xFF

- Byte 1 of member = (CallId >> 8) & 0xFF

- Byte 2 of member = CallId & 0xFF

During reading, each member ID is looked up in the address book to resolve the contact name. On write, member IDs are taken from the contact’s CallId. Members with all-zero ID bytes mark the end of the member list.

# 8. Channel Mode Block

| Property | Value |
| --- | --- |
| Read command | 0x10 (CmdReadChMode) |
| Write command | 0x30 (CmdWriteChMode) |
| Packet count | 64 |
| Payload size per packet | 1,024 bytes |
| Total payload | 65,536 bytes |
| Channels per packet | 16 |
| Bytes per channel record | 64 |
| Maximum areas | 16 |
| Maximum channels per area | 64 |
| Maximum total channel slots | 1,024 (16 areas × 64 channels) |
| Sequence number | 0–63 (increments per packet) |

Channel data is the largest single data region. Channels are numbered linearly from 0 to 1023 across all areas. The area index and in-area channel index are derived as: area = channel_index / 64, in_area_channel = channel_index % 64. Packet N carries channels N×16 through N×16+15.

A channel record is treated as empty if the first 4 bytes (RxFreq field) are all 0xFF or all 0x00.

## 8.1 Channel Record Structure (64 bytes)

| Offset (bytes) | Size | Field Name | Encoded Values / Notes |
| --- | --- | --- | --- |
| 0 – 3 | 4 | RxFreq | 32-bit little-endian integer, frequency in units of 10 Hz. e.g., 145,500,000 Hz (145.5 MHz) = 14,550,000 decimal = 0x00DE1B90, stored LE as 90 1B DE 00. |
| 4 – 7 | 4 | TxFreq | Same format as RxFreq. Set equal to RxFreq for simplex operation. |
| 8 – 9 | 2 | RxSubaudio | CTCSS/DCS sub-audio tone for receive. See Sub-Audio encoding (Section 8.2). |
| 10 – 11 | 2 | TxSubaudio | CTCSS/DCS sub-audio tone for transmit. Same encoding as RxSubaudio. |
| 12 | 1 | SignalingCode | Low nibble, modulo maxDtmfCodeGroupCount (15). Index into DTMF code group list (0 = none/off). |
| 13 | 1 | ChPttId | Low nibble, modulo 4. Per-channel PTT ID mode. Values: 0=OFF, 1=BOT, 2=EOT, 3=BOTH. |
| 14 | 1 | ChannelType flag A | Low nibble, modulo 2. 1=Analog (FM), 0=Digital (DMR). Combined with byte 15. |
| 15 | 1 | ChannelType flag B | Low nibble, modulo 2. When byte 14=0: 0=DMR Tier I (simplex), 1=DMR Tier II (repeater). When byte 14=1: 0=Wideband, 1=Narrow band. |
| 16 | 1 | TxPower | Low nibble, modulo 3. Transmit power level. Values: 0=Low, 1=Middle, 2=High. |
| 17 | 1 | Scramble | Low nibble, modulo 9. Analog scrambler type/code (0=Off, 1–8=scrambler codes). |
| 18 | 1 | Encryption | Low nibble, modulo 4. Digital encryption mode. Values: 0=None, 1=Basic, 2=Enhanced, 3=AES. |
| 19 | 1 | BusyLockout | Low nibble, modulo 2. 0=Off (transmit even if channel busy), 1=On (lock TX if busy). |
| 20 | 1 | ScanAdd | Low nibble, modulo 2. 0=No (exclude from scan list), 1=Yes (include in scan list). |
| 21 | 1 | TimeSlot | Low nibble, modulo 2. DMR time slot. 0=TS1, 1=TS2. Ignored on analog channels. |
| 22 | 1 | ColorCode | Low nibble, modulo 16. DMR color code 0–15. Ignored on analog channels. |
| 23 | 1 | RxGroup | Byte value, modulo (maxRxGroupCount+1) = modulo 33. 0=None, 1–32=Rx group index. |
| 24 | 1 | (reserved) | 0xFF |
| 25 | 1 | EncKey | Low nibble, modulo maxEncKeyCount (8). Index 0–7 into encryption key list (0=first key). |
| 26 | 1 | DmrMode | Low nibble, modulo 2. 0=Direct/Simplex (DMR Tier I), 1=Repeater (DMR Tier II). Affects how the radio handles call routing. |
| 27 | 1 | LearnFhss | Low nibble, modulo 2. Frequency Hopping Spread Spectrum auto-learn. 0=Off, 1=On. |
| 28 – 31 | 4 | FhssCode | FHSS hop sequence code. See FHSS encoding (Section 8.3). 0xFF in byte 31 indicates no FHSS code is set. |
| 32 – 43 | 12 | ChannelName | GB2312 encoded channel name string. Null (0x00) or 0xFF terminated. |
| 44 – 45 | 2 | Contact | Little-endian 16-bit index into address book (0=none/not set, 1-based index). |
| 46 – 63 | 18 | (reserved padding) | 0xFF |

## 8.2 Sub-Audio Encoding (2 bytes)

The 2-byte sub-audio field encodes three possible states: OFF, a CTCSS tone, or a DCS code.

| State | Byte 0 | Byte 1 | Decoded Value |
| --- | --- | --- | --- |
| OFF | 0x00 | 0x00 | No sub-audio / carrier squelch only |
| DCS code | DCS index + 1 (1–210) | 0x00 | DCS table entry at index (byte0 - 1). 105 normal (N) codes followed by 105 inverted (I) codes. e.g., 0x01 = D023N, 0x69 = D754N, 0x6A = D023I |
| CTCSS tone | Low byte of (freq × 10) | High byte of (freq × 10) | CTCSS frequency = (byte1 << 8 ¦ byte0) / 10 Hz. e.g., 88.5 Hz: 885 = 0x0375 → byte0=0x75, byte1=0x03 |

### Complete CTCSS Tone Table (50 standard tones)

Standard tones supported: 67.0, 69.3, 71.9, 74.4, 77.0, 79.7, 82.5, 85.4, 88.5, 91.5, 94.8, 97.4, 100.0, 103.5, 107.2, 110.9, 114.8, 118.8, 123.0, 127.3, 131.8, 136.5, 141.3, 146.2, 151.4, 156.7, 159.8, 162.2, 165.5, 167.9, 171.3, 173.8, 177.3, 179.9, 183.5, 186.2, 189.9, 192.8, 196.6, 199.5, 203.5, 206.5, 210.7, 218.1, 225.7, 229.1, 233.6, 241.8, 250.3, 254.1 Hz. Non-standard tones in range 60.0–260.0 Hz are also accepted.

## 8.3 FHSS Code Encoding (bytes 28–31)

The FHSS code is a 6-character uppercase hexadecimal string representing values 0x000000 to 0x7FFFFF. It is packed into 3 bytes (bytes 28–30) in reversed nibble order, with byte 31 used as a validity flag:

| Offset (bytes) | Size | Field Name | Encoded Values / Notes |
| --- | --- | --- | --- |
| 28 | 1 | FHSS code LSB byte | Packed: high nibble = hex digit 4 (0-indexed from MSB), low nibble = hex digit 5 |
| 29 | 1 | FHSS code mid byte | Packed: high nibble = hex digit 2, low nibble = hex digit 3 |
| 30 | 1 | FHSS code MSB byte | Packed: high nibble = hex digit 0, low nibble = hex digit 1 |
| 31 | 1 | FHSS validity flag | 0x00 = valid FHSS code present. 0xFF = no FHSS code (entry unused). |

Note: Example: FHSS code "1A2B3C" → digit0=1,1=A,2=2,3=B,4=3,5=C. byte30=(1<<4|A)=0x1A, byte29=(2<<4|B)=0x2B, byte28=(3<<4|C)=0x3C, byte31=0x00.

## 8.4 ChannelType Combined Decoding

The ChannelType field visible in the CPS is derived from bytes 14 and 15 together using the formula: ChannelType = (byte14 & 0x0F) % 2 + (byte15 & 0x0F) % 2.

| Byte 14 (nibble) | Byte 15 (nibble) | ChannelType value | Meaning |
| --- | --- | --- | --- |
| 0 | 0 | 0 | Analog FM |
| 1 | 0 | 1 | Digital DMR Tier I (simplex) |
| 1 | 1 | 2 | Digital DMR Tier II (repeater) |

# 9. VFO Mode Block

| Property | Value |
| --- | --- |
| Read command | 0x11 (CmdReadVfoMode) |
| Write command | 0x31 (CmdWriteVfoMode) |
| Packet count | 1 |
| Payload size | 128 bytes |
| VFO banks | 2 (Bank A at bytes 0–63; Bank B at bytes 64–127) |
| Default RxFreq Bank A | 136.12500 MHz |
| Default RxFreq Bank B | 400.12500 MHz |

The VFO block carries two VFO bank definitions, each 64 bytes. The layout is nearly identical to a channel record with three differences: the StepFreq field replaces LearnFhss (byte 27), the Contact field is at bytes 32–33 rather than 44–45, and there is no ChannelName, ScanAdd, or ChPttId field.

If a frequency read from the radio is all 0x00 or all 0xFF, the CPS substitutes the bank’s default frequency automatically.

## 9.1 VFO Bank Structure (64 bytes)

| Offset (bytes) | Size | Field Name | Encoded Values / Notes |
| --- | --- | --- | --- |
| 0 – 3 | 4 | RxFreq | 32-bit LE integer, 10 Hz units. Same encoding as channel. Default A=136.125 MHz, B=400.125 MHz. |
| 4 – 7 | 4 | TxFreq | Same format as RxFreq. |
| 8 – 9 | 2 | RxSubaudio | CTCSS/DCS sub-audio. Same encoding as channel (Section 8.2). Default: OFF. |
| 10 – 11 | 2 | TxSubaudio | Same encoding as RxSubaudio. Default: OFF. |
| 12 | 1 | SignalingCode | Low nibble, modulo 15. DTMF code group index. 0=none. |
| 13 | 1 | (unused) | 0xFF. Note: ChPttId is not stored in VFO mode. |
| 14 | 1 | ChannelType flag A | Low nibble. Same as channel byte 14. |
| 15 | 1 | ChannelType flag B | Low nibble. Same as channel byte 15. |
| 16 | 1 | TxPower | Low nibble, modulo 3. 0=Low, 1=Medium, 2=High. |
| 17 | 1 | Scramble | Low nibble, modulo 9. 0=Off, 1–8=scrambler codes. |
| 18 | 1 | Encryption | Low nibble, modulo 4. 0=None, 1=Basic, 2=Enhanced, 3=AES. |
| 19 | 1 | BusyLockout | Low nibble, modulo 2. 0=Off, 1=On. |
| 20 | 1 | (unused) | 0xFF. Note: ScanAdd is not applicable to VFO. |
| 21 | 1 | TimeSlot | Low nibble, modulo 2. 0=TS1, 1=TS2. |
| 22 | 1 | ColorCode | Low nibble, modulo 16. DMR color code 0–15. |
| 23 | 1 | RxGroup | Byte value, modulo 33. 0=None, 1–32=Rx group index. |
| 24 | 1 | (unused) | 0xFF |
| 25 | 1 | EncKey | Low nibble, modulo 8. Encryption key index. |
| 26 | 1 | DmrMode | Low nibble, modulo 2. 0=Simplex, 1=Repeater. |
| 27 | 1 | StepFreq | Low nibble, modulo 8. Tuning step size. See value table below. |
| 28 – 31 | 4 | (unused) | 0xFF. FHSS is not applicable to VFO mode. |
| 32 – 33 | 2 | Contact | Little-endian 16-bit address book index. 0=none. |
| 34 – 63 | 30 | (reserved padding) | 0xFF |

### StepFreq values (byte 27, low nibble)

| Value (decimal) | Meaning |
| --- | --- |
| 0 | 2.5 kHz |
| 1 | 5.0 kHz |
| 2 | 6.25 kHz |
| 3 | 10.0 kHz |
| 4 | 12.5 kHz |
| 5 | 20.0 kHz |
| 6 | 25.0 kHz |
| 7 | 50.0 kHz |

# 10. Optional Functions Block

| Property | Value |
| --- | --- |
| Read command | 0x12 (CmdReadOptionalFun) |
| Write command | 0x32 (CmdWriteOptionalFun) |
| Packet count | 1 |
| Payload size | 64 bytes (two 32-byte sub-regions) |

This block contains all global radio settings and programmable button assignments. It is split into two 32-byte halves. The first half covers operational parameters; the second covers timing, key assignments, and digital-mode settings.

## 10.1 Optional Functions Part 1 (bytes 0–31)

| Offset (bytes) | Size | Field Name | Encoded Values / Notes |
| --- | --- | --- | --- |
| 0 | 1 | AnalogSql | Low nibble, modulo 10. Analog squelch level. 0=OFF, 1–9=Level 1–9. Default: 3. |
| 1 | 1 | PowerSave | Low nibble, modulo 5. Battery saver duty cycle. 0=OFF, 1=1:1, 2=1:2, 3=1:3, 4=1:4. Default: 1 (1:1). |
| 2 | 1 | Vox | Low nibble, modulo 10. VOX sensitivity. 0=OFF, 1–9=Level 1–9. Default: 0 (OFF). |
| 3 | 1 | AutoBacklight | Low nibble, modulo 9. Backlight auto-off timer. 0=Bright (always on), 1=5s, 2=10s, 3=15s, 4=20s, 5=30s, 6=1min, 7=2min, 8=3min. Default: 5 (30s). |
| 4 | 1 | Tdr | Low nibble, modulo 2. Dual Watch (TDR). 0=OFF, 1=ON. Default: 0. |
| 5 | 1 | Tot | Low nibble, modulo 6. TX Time-Out Timer. 0=OFF, 1=30s, 2=60s, 3=120s, 4=240s, 5=480s. Default: 0. |
| 6 | 1 | Beep | Low nibble, modulo 2. Keypad beep. 0=OFF, 1=ON. Default: 1. |
| 7 | 1 | Voice | Low nibble, modulo 2. Voice prompt. 0=OFF, 1=ON. Default: 1. |
| 8 | 1 | Language | Low nibble, modulo 2. UI language. 0=English, 1=Chinese. Default: 1 (English in source). |
| 9 | 1 | SideTone | Low nibble, modulo 4. Side tone mode. 0=OFF, 1=DT-ST (DTMF side tone), 2=ANI-ST (ANI side tone), 3=DT+ANI. Default: 0. |
| 10 | 1 | ScanMode | Low nibble, modulo 3. Scan resume mode. 0=TO (time-operated), 1=CO (carrier-operated), 2=SE (search). Default: 1. |
| 11 | 1 | PttId | Low nibble, modulo 4. Global PTT ID. 0=OFF, 1=BOT, 2=EOT, 3=BOTH. Default: 0. |
| 12 | 1 | IdDelayTime | Low nibble, modulo 7. PTT ID send delay. 0=0ms, 1=100ms, 2=200ms, 3=400ms, 4=600ms, 5=800ms, 6=1000ms. Default: 4 (600ms). |
| 13 | 1 | DisplayModeA | Low nibble, modulo 3. Band A display. 0=Name, 1=Freq, 2=CH (channel number). Default: 0. |
| 14 | 1 | DisplayModeB | Low nibble, modulo 3. Band B display. 0=Name, 1=Freq, 2=CH. Default: 0. |
| 15 | 1 | (reserved) | 0xFF |
| 16 | 1 | AutoLock | Low nibble, modulo 4. Auto keyboard lock timer. 0=OFF, 1=5s, 2=10s, 3=15s. Default: 0. |
| 17 | 1 | SosMode | Low nibble, modulo 3. SOS/emergency mode. 0=On Site (local alarm only), 1=Send Sound (TX alarm tone), 2=Send Code (TX DTMF code). Default: 0. |
| 18 | 1 | AlarmSound | Low nibble, modulo 2. Alarm sound enable. 0=OFF, 1=ON. Default: 1. |
| 19 | 1 | TdrTx | Low nibble, modulo 3. Dual Watch TX priority band. 0=OFF, 1=Band-A, 2=Band-B. Default: 0. |
| 20 | 1 | TailClear | Low nibble, modulo 2. Analog tail tone clear (reverse burst). 0=OFF, 1=ON. Default: 1. |
| 21 | 1 | RptClearTail | Low nibble, modulo 11. Repeater tail-clear tone duration in ms. 0=0ms, 1=100ms, ..., 10=1000ms. Default: 0. |
| 22 | 1 | RptDetectTail | Low nibble, modulo 11. Repeater tail-detect threshold. Same ms scale as byte 21. Default: 0. |
| 23 | 1 | TxOverSound | Low nibble, modulo 2. TX timeout warning sound. 0=OFF, 1=ON. Default: 0. |
| 24 | 1 | CurWorkMode | Low nibble, modulo 2. Active work band. 0=Band-A, 1=Band-B. Default: 0. |
| 25 | 1 | FmRadio | Low nibble, modulo 2. FM broadcast radio enable. 0=ON (allowed), 1=OFF (disabled). Default: 0. |
| 26 | 1 | WorkModeA + WorkModeB | High nibble = WorkModeB, Low nibble = WorkModeA. Each nibble: 0=VFO (frequency mode), 1=Channel mode. Default: both 0. |
| 27 | 1 | KeyLock | Low nibble, modulo 2. Keyboard lock state. 0=OFF (unlocked), 1=ON (locked). Default: 0. |
| 28 | 1 | BootScreen | Low nibble, modulo 2. Boot screen type. 0=Logo (preset graphic), 1=Voltage (show battery voltage). Default: 0. |
| 29 | 1 | (reserved) | 0xFF |
| 30 | 1 | RTone | Low nibble, modulo 4. Roger beep tone frequency. 0=1000Hz, 1=1450Hz, 2=1750Hz, 3=2100Hz. Default: 2 (1750Hz). |
| 31 | 1 | TxStartSound | Low nibble, modulo 2. Sound when TX begins. 0=OFF, 1=ON. Default: 0. |

## 10.2 Optional Functions Part 2 (bytes 32–63)

| Offset (bytes) | Size | Field Name | Encoded Values / Notes |
| --- | --- | --- | --- |
| 32 | 1 | VoxDelayTime | Low nibble, modulo 16. VOX release delay time. 0=0.5s, 1=0.6s, 2=0.7s, 3=0.8s, 4=0.9s, 5=1.0s, 6=1.1s, 7=1.2s, 8=1.3s, 9=1.4s, 10=1.5s, 11=1.6s, 12=1.7s, 13=1.8s, 14=1.9s, 15=2.0s. Default: 5 (1.0s). |
| 33 | 1 | MenuAutoQuit | Low nibble, modulo 11. Menu auto-exit timer. 0=5s, 1=10s, 2=15s, 3=20s, 4=25s, 5=30s, 6=35s, 7=40s, 8=45s, 9=50s, 10=60s. Default: 1 (10s). |
| 34 | 1 | DigitalSql | Low nibble, modulo 10. Digital (DMR) squelch level. 0=OFF, 1–9=Level 1–9. Default: 3. |
| 35 – 38 | 4 | (reserved) | 0xFF |
| 39 | 1 | SteFreq | Low nibble, modulo 2. Sub-Tone Elimination frequency. 0=55 Hz, 1=62.5 Hz. Default: 0. |
| 40 | 1 | WeatherCh | Low nibble, modulo 10. NOAA/Weather channel preset. 0–9 = channels 1–10. Default: 0. |
| 41 – 42 | 2 | (reserved) | 0xFF |
| 43 | 1 | TopKey1S | Low nibble, modulo 7. Top button short-press function. 0=Radio, 1=Moni, 2=Scan, 3=Search, 4=SOS, 5=Noaa, 6=ScanQT. Default: 0. |
| 44 | 1 | SideKey2S | Low nibble, modulo 8. Side key 2 short-press function. 0=Radio, 1=Moni, 2=Scan, 3=Search, 4=SOS, 5=Noaa, 6=ScanQT, 7=PTTB. Default: 1 (Moni). |
| 45 | 1 | SideKey2L | Low nibble, modulo 7. Side key 2 long-press function. Same 7 values as TopKey1S. Default: 2 (Scan). |
| 46 | 1 | SideKey3S | Low nibble, modulo 7. Side key 3 short-press function. Same 7 values as TopKey1S. Default: 3 (Search). |
| 47 | 1 | SideKey3L | Low nibble, modulo 7. Side key 3 long-press function. Same 7 values as TopKey1S. Default: 4 (SOS). |
| 48 – 49 | 2 | (reserved) | 0xFF |
| 50 | 1 | KeepCallTime | Bits 4:0 (& 0x1F), modulo 20. Hang-time after call ends before scanner resumes. 0=1s, 1=2s, ..., 19=20s. Default: 4 (5s). |
| 51 | 1 | (reserved) | 0xFF |
| 52 | 1 | TdrRecoveryTime | Low nibble, modulo 11. Dual Watch recovery time. 0=0.5s, 1=1s, 2=2s, 3=3s, 4=4s, 5=5s, 6=6s, 7=7s, 8=8s, 9=9s, 10=10s. Default: 3 (3s). |
| 53 – 63 | 11 | (reserved) | 0xFF |

### Programmable Key Function Codes

| Code | Short label | Function |
| --- | --- | --- |
| 0 | Radio | FM broadcast radio toggle |
| 1 | Moni | Monitor / squelch open |
| 2 | Scan | Channel scan start/stop |
| 3 | Search | Frequency search/scan |
| 4 | SOS | SOS/emergency alert |
| 5 | Noaa | NOAA weather channel |
| 6 | ScanQT | Scan sub-tone (CTCSS/DCS scan) |
| 7 | PTTB | PTT for Band B (Side key 2 short only) |

# 11. Basic Info Block

| Property | Value |
| --- | --- |
| Read command | 0x19 (CmdReadBasicInfo) |
| Write command | 0x39 (CmdWriteBasicInfo) |
| Packet count | 1 |
| Payload size | 64 bytes |
| Write condition | Only sent if WriteModelNameAndId flag is true in CPS |

The basic info block stores the radio’s model identity. Only two fields within the 64-byte payload are actively used. The write of this block is conditional: the CPS only transmits it if the user has explicitly enabled the WriteModelNameAndId option, making it suitable for production-line model stamping without affecting normal programming workflows.

## 11.1 Basic Info Payload (64 bytes)

| Offset (bytes) | Size | Field Name | Encoded Values / Notes |
| --- | --- | --- | --- |
| 0 – 7 | 8 | (reserved prefix) | 0xFF |
| 8 – 19 | 12 | ModelName | GB2312 encoded model name string. Null (0x00) or 0xFF terminated. Maximum 12 bytes. Default: "DMR". |
| 20 – 27 | 8 | ModelId | ASCII decimal string, zero-padded to exactly 8 characters. e.g., ModelId=1 → "00000001" → bytes: 30 30 30 30 30 30 30 31. Range: 1–16,776,415. |
| 28 – 63 | 36 | (reserved suffix) | 0xFF |

Note: ModelId is stored as ASCII text, not as a binary integer. When reading, the CPS parses the 8 ASCII bytes as a decimal number using int.TryParse. If SelfIncrementModelId is enabled in the CPS, the ModelId is automatically incremented after each successful write, supporting sequential device serialisation.

# 12. End Session Block

| Property | Value |
| --- | --- |
| Command | 0x01 (CmdOver) |
| Packet count | 1 |
| Payload size | 2 bytes |
| Sequence number | 0 (always) |

Signals the end of the programming session. After receiving this packet and sending a valid response, the radio exits programming mode and returns to normal operation. Failure to send this command (e.g., due to a CPS crash or forced disconnect) will leave the radio in programming mode until it times out internally or is power-cycled.

| Offset (bytes) | Size | Field Name | Encoded Values / Notes |
| --- | --- | --- | --- |
| 0 – 1 | 2 | End marker | Two bytes: 00 00 |

# Appendix A: Cross-Reference Table

This table shows how each data field in the CPS is stored in memory versus how it is encoded on the wire.

| Field | CPS Internal Type | Wire Encoding | Block |
| --- | --- | --- | --- |
| Frequency | string (MHz, e.g. "145.5000") | 32-bit LE int, units of 10 Hz | Channel, VFO |
| CTCSS tone | string (Hz, e.g. "88.5") | 16-bit LE int = tone × 10 | Channel, VFO |
| DCS code | string (e.g. "D023N") | byte = table index + 1, then 0x00 | Channel, VFO |
| Channel name | string (GB2312) | GB2312 bytes, 0xFF padded, 12 bytes | Channel |
| Contact name | string (GB2312) | GB2312 bytes, 0xFF padded, 10 bytes | Address Book |
| Group name | string (GB2312) | GB2312 bytes, 0xFF padded, 12 bytes | Rx Group |
| DMR ID (CallId) | int (1 – 16,777,215) | 4 bytes: 0x00 + 3-byte BE integer | Address Book |
| DMR ID (member) | int | 3-byte BE integer only | Rx Group |
| Enc key | string (hex digits) | Each char = nibble index 0–15 in hex alpha | Enc Key |
| DTMF code | string (DTMF digits) | Each char = nibble index 0–15 in DTMF alpha | DTMF |
| DTMF ID | string (DTMF digits) | Each char = nibble index 0–15, 5 bytes max | DTMF |
| FHSS code | string (6 hex chars) | 3 bytes packed BCD reversed, byte31=0x00 | Channel |
| Model name | string (ASCII/GB2312) | GB2312 bytes, 0xFF padded, 12 bytes | Basic Info |
| Model ID | int | 8-byte ASCII decimal string, zero-padded | Basic Info |
| Enum settings | int (0-based index) | Single byte, low nibble used, range-clamped | Optional Fun |
| Contact index | int (0=none) | 16-bit LE (Channel: bytes 44-45; VFO: bytes 32-33) | Channel, VFO |
| Rx Group index | int (0=none, 1-32=group) | Single byte value | Channel, VFO |

# Appendix B: Packet Size Summary

| Block | Cmd (R/W) | Pkts | Payload bytes/pkt | Total payload | Frame bytes/pkt | Total on wire |
| --- | --- | --- | --- | --- | --- | --- |
| Handshake | 0x02 | 1 | 15 | 15 | 23 | 23 |
| Password | 0x05 | 1 | 6 | 6 | 14 | 14 |
| Version | 0x46 | 1 | 128 | 128 | 136 | 136 |
| DTMF | 0x16/0x36 | 1 | 272 | 272 | 280 | 280 |
| Enc Keys | 0x15/0x35 | 1 | 264 | 264 | 272 | 272 |
| Address Book | 0x13/0x33 | 80 | 800 | 64,000 | 808 | 64,640 |
| Rx Groups | 0x14/0x34 | 4 | 1,024 | 4,096 | 1,032 | 4,128 |
| Channels | 0x10/0x30 | 64 | 1,024 | 65,536 | 1,032 | 66,048 |
| VFO Mode | 0x11/0x31 | 1 | 128 | 128 | 136 | 136 |
| Optional Fun | 0x12/0x32 | 1 | 64 | 64 | 72 | 72 |
| Basic Info | 0x19/0x39 | 1 | 64 | 64 | 72 | 72 |
| End Session | 0x01 | 1 | 2 | 2 | 10 | 10 |
| TOTALS |  | 157 | — | 134,575 | — | 135,831 |

Note: 'Frame bytes/pkt' = payload + 8 bytes of framing overhead (SOF + cmd + seq(2) + len(2) + CRC(2)). 'Total on wire' = frame bytes × packet count. Actual session total also includes radio response packets (same structure), approximately doubling the wire volume.

End of Data Structure Reference