RT-5D DMR Transceiver

Programming Protocol Reverse-Engineering Report

Derived from CPS Source Code Analysis (JJCC-888DMR_CPS)

February 2026

# 1. Overview

This document presents a detailed analysis of the programming protocol used by the RT-5D DMR transceiver (also identified internally as the JJCC-888DMR). The analysis is derived entirely from the Customer Programming Software (CPS) source code supplied with the radio, specifically the C# .NET project "JJCC-888DMR_CPS".

The radio uses a custom, binary, framed serial protocol over a USB-serial adapter to exchange configuration data with a host PC. The protocol is symmetric in structure: both the PC and radio use the same packet framing. All configuration is transferred in a fixed sequence of typed data blocks, each covering a specific memory region or feature set within the radio.

# 2. Physical & Serial Layer

The CPS establishes the serial connection using the following fixed parameters, as seen in MainForm.cs:

| Parameter | Value |
| --- | --- |
| Baud Rate | 115200 |
| Data Bits | 8 |
| Parity | None (0) |
| Stop Bits | 1 |
| Flow Control | None (default) |
| Interface | USB-Serial (COM port, user-selectable) |

Note: The baud rate of 115200 is hardcoded in all three communication call sites within MainForm.cs. There is no user-configurable baud rate option in the CPS.

# 3. Packet Framing

Every message exchanged between the CPS and radio is encapsulated in a fixed-format binary frame. The structure is defined by the DataHandleHelper class. The frame is always transmitted and received in this exact layout:

| Offset (bytes) | Field | Size (bytes) | Value / Notes |
| --- | --- | --- | --- |
| 0 | Frame Header (SOF) | 1 | 0xA5 (165 decimal) - always this value |
| 1 | Command Byte | 1 | Identifies the operation (see Command Table) |
| 2-3 | Sequence / Page ID | 2 | Big-endian 16-bit. Used as packet/page counter |
| 4-5 | Payload Length | 2 | Big-endian 16-bit. Number of payload bytes (N) |
| 6 ... 6+N-1 | Payload Data | N | The data for this packet |
| 6+N | CRC High Byte | 1 | CRC-16/CCITT, upper byte |
| 6+N+1 | CRC Low Byte | 1 | CRC-16/CCITT, lower byte |

Total frame size = 6 (header) + N (payload) + 2 (CRC) = N + 8 bytes.

## 3.1 Start-of-Frame Detection (Receive Side)

The CPS receive state machine is three-stage:

- Stage 1 (ReceiveDataOne): Read bytes one at a time until 0xA5 is found. This byte marks the start of a frame.

- Stage 2 (ReceiveDataTwo): Read the next 5 bytes (bytes 1-5: command, sequence high, sequence low, length high, length low). Extract the payload length N from bytes 4-5.

- Stage 3 (HandleData): Wait until N + 2 bytes are available (payload + CRC), then read and validate.

## 3.2 CRC Algorithm

The CRC is CRC-16/CCITT (polynomial 0x1021, no initial value preset to 0xFFFF - the code XORs starting from 0). The CRC is computed over bytes 1 through 5+N (i.e., all frame fields except the SOF byte and the CRC itself). The algorithm from DataHandleHelper.CrcValidation():

int crc = 0;

for each byte in dat[offset .. offset+count-1]:

crc ^= (byte << 8)

for 8 iterations:

if (crc & 0x8000) != 0: crc = (crc << 1) ^ 0x1021

else:                  crc = (crc << 1)

crc = crc & 0xFFFF

CRC covers: cmd (byte 1) + seq_hi + seq_lo + len_hi + len_lo + payload (bytes 1 through 5+N).

Note: A reply with command byte 0xEE (238 decimal) is treated as a NAK/error response and is silently ignored by the receive handler.

# 4. Command Set

The following command codes are defined in DataComCommandType.cs. The SOF byte (0xA5) is also defined there as 'FrameHeader'.

| Command Name | Hex Value | Dec | Direction | Description |
| --- | --- | --- | --- | --- |
| CmdHandshake | 0x02 | 2 | Host -> Radio | Initial handshake, sends 15-byte ASCII string |
| CmdCheckPwd | 0x05 | 5 | Host -> Radio | Send 6-byte terminal password for auth |
| CmdGetVersion | 0x46 | 70 | Host -> Radio | Request 128-byte version block from radio |
| CmdOver | 0x01 | 1 | Host -> Radio | Session end / programming complete |
| CmdReadChMode | 0x10 | 16 | Host -> Radio | Read channel mode data block |
| CmdReadVfoMode | 0x11 | 17 | Host -> Radio | Read VFO mode data block |
| CmdReadOptionalFun | 0x12 | 18 | Host -> Radio | Read optional functions block |
| CmdReadAddrBook | 0x13 | 19 | Host -> Radio | Read address book (contacts) block |
| CmdReadRxGroup | 0x14 | 20 | Host -> Radio | Read receive group list block |
| CmdReadEncKey | 0x15 | 21 | Host -> Radio | Read encryption keys block |
| CmdReadDtmf | 0x16 | 22 | Host -> Radio | Read DTMF settings block |
| CmdReadBasicInfo | 0x19 | 25 | Host -> Radio | Read basic info (model name/ID) block |
| CmdWriteChMode | 0x30 | 48 | Host -> Radio | Write channel mode data block |
| CmdWriteVfoMode | 0x31 | 49 | Host -> Radio | Write VFO mode data block |
| CmdWriteOptionalFun | 0x32 | 50 | Host -> Radio | Write optional functions block |
| CmdWriteAddrBook | 0x33 | 51 | Host -> Radio | Write address book block |
| CmdWriteRxGroup | 0x34 | 52 | Host -> Radio | Write receive group list block |
| CmdWriteEncKey | 0x35 | 53 | Host -> Radio | Write encryption keys block |
| CmdWriteDtmf | 0x36 | 54 | Host -> Radio | Write DTMF settings block |
| CmdWriteBasicInfo | 0x39 | 57 | Host -> Radio | Write basic info (model/ID) - optional |
| (NAK/Error) | 0xEE | 238 | Radio -> Host | Radio error response; CPS ignores and retries |

Note: Additional image-related commands are defined (CmdImageHandshake=0x02, CmdIntoImageFlashEditMode=0x08, CmdSetImageAddress=0x03, CmdEraseImageOldData=0x04, CmdWriteImageNewData=0x57, CmdImageOver=0x06) for the boot screen image programming feature, which follows a separate flow not detailed in this document.

# 5. Full Protocol Sequence

The communication always proceeds in a fixed, strict sequence regardless of direction (read or write). Each step is a request-response pair. Below is the complete ordered sequence:

| Step # | Phase Name | Command | Sequence/Pages | Payload Size (bytes) | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Handshake | 0x02 | 0 | 15 | ASCII: "PROGRAMJC8810DU" |
| 2 | Check Password | 0x05 | 0 | 6 | 6 bytes: FF FF FF FF FF FF (default/blank) |
| 3 | Get Version | 0x46 | 0 | 128 | 128-byte placeholder; radio returns version info |
| 4 | DTMF | 0x16 / 0x36 | 0 | 272 | 1 packet only |
| 5 | Encryption Keys | 0x15 / 0x35 | 0 | 264 | 1 packet only |
| 6 | Address Book | 0x13 / 0x33 | 0-79 | 800 ea. | 80 packets x 800 bytes = 64,000 bytes total |
| 7 | Rx Group List | 0x14 / 0x34 | 0-3 | 1024 ea. | 4 packets x 1024 bytes = 4,096 bytes total |
| 8 | Channel Mode | 0x10 / 0x30 | 0-63 | 1024 ea. | 64 packets x 1024 bytes = 65,536 bytes total |
| 9 | VFO Mode | 0x11 / 0x31 | 0 | 128 | 1 packet (2 VFOs x 64 bytes each) |
| 10 | Optional Functions | 0x12 / 0x32 | 0 | 64 | 1 packet |
| 11 | Basic Info | 0x19 / 0x39 | 0 | 64 | 1 packet (write is conditional - see notes) |
| 12 | End Session | 0x01 | 0 | 2 | 2 zero bytes; radio exits programming mode |

Note: For a READ session, the CPS sends Read commands (0x10-0x19) and receives the payload in the radio's response. For a WRITE session, the CPS sends Write commands (0x30-0x39) carrying the payload. In both cases the same packet structure applies.

Note: The BasicInfo write step (Step 11) is conditional: it only executes if the flag BasicInfoData.WriteModelNameAndId is true. This flag appears to be set only when writing model identity to the radio is explicitly desired.

## 5.1 Timeout and Retry Mechanism

After transmitting a packet, the CPS starts a software timer with the following parameters:

- Timer interval: 200 ms per tick

- Timeout counter: 5 ticks (i.e., ~1 second timeout before flagging a retry)

- Maximum retry attempts: 3

On timeout, the CPS flushes any pending bytes from the receive buffer, retransmits the last packet exactly as sent, and decrements the retry counter. After 3 failed retries the session is aborted and CommFail is returned.

# 6. Data Block Formats

What follows is a byte-level description of each data block payload. All multi-byte integers are little-endian unless otherwise noted. Unused/padding bytes are filled with 0xFF. A leading 0xFF in a field generally indicates "empty" or "not programmed".

## 6.1 Handshake Block (15 bytes)

The handshake payload is the ASCII string PROGRAMJC8810DU, which encodes to exactly 15 bytes:

50 52 4F 47 52 41 4D 4A 43 38 38 31 30 44 55

The radio is expected to reply with the same command byte (0x02) and a valid response payload. The CPS does not validate the content of the handshake reply, only that a valid framed response is received.

## 6.2 Password Block (6 bytes)

The password block sends 6 bytes representing the terminal password. The default/blank password is all 0xFF:

FF FF FF FF FF FF

The radio validates this against its stored password. A NAK (0xEE) response would indicate a wrong password, though the CPS does not implement a specific password-error handler beyond the general NAK ignore.

## 6.3 Version Block (128 bytes)

The CPS sends 128 zero bytes as a placeholder. The radio replies with 128 bytes of version/identity information. The CPS does not parse this block beyond acknowledging receipt - it is informational only and used to confirm the radio type before proceeding.

## 6.4 DTMF Block (272 bytes, 1 packet)

The DTMF block is split into two sub-regions:

| Offset | Size | Field | Encoding |
| --- | --- | --- | --- |
| 0-4 | 5 | Current ID | Each nibble = DTMF digit index into "0123456789ABCD*#"; 0xFF = empty |
| 5 | 1 | (reserved/pad) | 0xFF |
| 6 | 1 | PTT ID Mode | Low nibble: 0=Off, 1=BOT, 2=EOT, 3=Both |
| 7 | 1 | DTMF Duration | Low nibble: 0-4 (index into duration table) |
| 8 | 1 | DTMF Interval | Low nibble: 0-4 (index into interval table) |
| 9-31 | 23 | (padding) | 0xFF |
| 32-271 | 240 | Code Group List | 15 groups x 16 bytes each |

Each Code Group entry (16 bytes) layout:

| Offset within entry | Size | Field | Encoding |
| --- | --- | --- | --- |
| 0-5 | 6 | DTMF Code | Each byte = digit index into "0123456789ABCD*#"; 0xFF = end/empty |
| 6-15 | 10 | (padding) | 0xFF |

## 6.5 Encryption Key Block (264 bytes, 1 packet)

Supports up to 8 encryption key entries, each 33 bytes:

| Offset within entry | Size | Field | Encoding |
| --- | --- | --- | --- |
| 0 | 1 | Algorithm | Low nibble: 0=None, 1=Basic, 2=Enhanced |
| 1-32 | 32 | Key Data | Each byte = hex digit index into "0123456789ABCDEF"; 0xFF = empty |

Total: 8 x 33 = 264 bytes. Entries marked 0xFF in byte 0 and byte 1 are treated as empty.

## 6.6 Address Book (800 bytes/packet, 80 packets = 4,000 contacts max)

The address book is the largest data region, transmitted in 80 consecutive packets. Each packet carries data for 50 contacts. Contact record layout (16 bytes each):

| Offset | Size | Field | Encoding |
| --- | --- | --- | --- |
| 0 | 1 | Call Type | Low nibble: 0=Group, 1=Private, 2=All Call |
| 1 | 1 | (zero byte) | Always 0x00 for valid entries |
| 2 | 1 | Call ID byte 2 (MSB) | Big-endian 24-bit DMR ID (bytes 2-4) |
| 3 | 1 | Call ID byte 1 |  |
| 4 | 1 | Call ID byte 0 (LSB) |  |
| 5-14 | 10 | Contact Name | GB2312 encoded string, null/0xFF terminated |
| 15 | 1 | (padding) | 0xFF |

An entry with bytes 0, 1, or 5 == 0xFF is considered unoccupied. The Call ID is validated to be within the DMR address range 1-16776415 (0x000001 to 0xFFFFFF).

## 6.7 Receive Group List (1024 bytes/packet, 4 packets = 32 groups max)

Each packet carries data for 8 receive groups. Each group occupies 128 bytes:

| Offset within group entry | Size | Field | Encoding |
| --- | --- | --- | --- |
| 0-95 | 96 | Member ID List | Up to 32 members x 3 bytes each (24-bit DMR ID, big-endian) |
| 96-107 | 12 | Group Name | GB2312 encoded, null/0xFF terminated |
| 108-127 | 20 | (padding) | 0xFF |

Member IDs of 0x000000 indicate no further members. Each member is cross-referenced to the address book to resolve the contact name. An entry is skipped if byte 96 == 0xFF (name field empty).

## 6.8 Channel Mode (1024 bytes/packet, 64 packets = 1024 channels across up to 10 areas)

Channel data is the most complex block. Each packet carries 16 channels. Each channel occupies 64 bytes. The channel numbering is linear: packet N carries channels N*16 through N*16+15. Channels are mapped to areas using the formula: area = channel_index / channels_per_area.

Channel record layout (64 bytes):

| Offset | Size | Field | Encoding / Notes |
| --- | --- | --- | --- |
| 0-3 | 4 | Rx Frequency | Little-endian 32-bit integer x 10 Hz (e.g., 0x002DC6C0 = 146.520 MHz). Stored as raw Hz/10 integer. |
| 4-7 | 4 | Tx Frequency | Same format as Rx Frequency |
| 8-9 | 2 | Rx Sub-Audio | See Sub-Audio encoding below |
| 10-11 | 2 | Tx Sub-Audio | See Sub-Audio encoding below |
| 12 | 1 | Signaling Code | Low nibble: index into DTMF code group list (0=none) |
| 13 | 1 | PTT ID | Low nibble: 0=Off, 1=BOT, 2=EOT, 3=Both |
| 14 | 1 | Channel Type flag 1 | Low nibble: 0=Analog, 1=Digital |
| 15 | 1 | Channel Type flag 2 | Low nibble: 0=DMR Tier I, 1=DMR Tier II (only if byte 14=1) |
| 16 | 1 | Tx Power | Low nibble: 0=Low, 1=Medium, 2=High |
| 17 | 1 | Scramble | Low nibble: 0-8 (scrambler type/code) |
| 18 | 1 | Encryption | Low nibble: 0=None, 1=Basic, 2=Enhanced, 3=AES |
| 19 | 1 | Busy Lockout | Low nibble: 0=Off, 1=On |
| 20 | 1 | Scan Add | Low nibble: 0=No, 1=Yes |
| 21 | 1 | Time Slot | Low nibble: 0=TS1, 1=TS2 |
| 22 | 1 | Color Code | Low nibble: 0-15 |
| 23 | 1 | Rx Group | Byte value: 0=None, 1-32=Rx group index |
| 24 | 1 | (reserved) | 0xFF |
| 25 | 1 | Enc Key Index | Low nibble: index into encryption key list |
| 26 | 1 | DMR Mode | Low nibble: 0=Simplex, 1=Repeater |
| 27 | 1 | Learn FHSS | Low nibble: 0=Off, 1=On (Frequency Hopping Spread Spectrum) |
| 28-31 | 4 | FHSS Code | 6 hex digits packed BCD, byte 31 = 0x00 if valid, 0xFF if unused |
| 32-43 | 12 | Channel Name | GB2312 encoded, null/0xFF terminated |
| 44-45 | 2 | Contact Index | Little-endian 16-bit: index into address book (0=none) |
| 46-63 | 18 | (padding) | 0xFF |

### 6.8.1 Frequency Encoding

Frequencies are stored as 32-bit little-endian integers representing the frequency in units of 10 Hz. For example, 146.520 MHz is stored as 14652000 (decimal) = 0x00DFC1A0, stored little-endian as A0 C1 DF 00.

The CPS validates frequencies against: VHF 18-300 MHz (exclusive of 300), UHF 300-1000 MHz. The step granularity is 10 Hz.

### 6.8.2 Sub-Audio (CTCSS/DCS) Encoding

The 2-byte sub-audio field encodes both CTCSS tones and DCS codes:

- OFF: Both bytes 0x00

- DCS code: Byte 0 = (DCS index + 1), Byte 1 = 0x00. The DCS table contains standard codes in order (D023N, D025N, etc.)

- CTCSS tone: Little-endian 16-bit integer = tone frequency x 10 (e.g., 88.5 Hz = 885 = 0x0375, stored as 75 03). Reconstructed by inserting decimal point before last digit.

### 6.8.3 FHSS Code Encoding

FHSS (Frequency Hopping Spread Spectrum) codes are 6 hex-digit values packed into 3 bytes (bytes 28-30), with byte 31 = 0x00 to signal a valid entry. The digits are stored in reverse order: byte 30 holds digits 1-2 (most significant), byte 29 holds digits 3-4, byte 28 holds digits 5-6 (least significant). Byte 28 ORed with the packed value.

## 6.9 VFO Mode Block (128 bytes, 1 packet)

The VFO block contains two VFO banks (A and B), each 64 bytes, using a layout nearly identical to the channel record. Differences from the channel format:

- Byte 27 is Step Frequency (0-7 index) rather than Learn FHSS

- Bytes 32-33 hold Contact index (little-endian 16-bit) instead of at bytes 44-45

- No ScanAdd, no ChPttId field (byte 13 unused)

- No channel name field

VFO A occupies bytes 0-63, VFO B occupies bytes 64-127. If the frequency bytes are all 0xFF or all 0x00, the CPS substitutes a default frequency.

## 6.10 Optional Functions Block (64 bytes, 1 packet)

This block is split into two 32-byte sub-regions. Part 1 (bytes 0-31):

| Offset | Field | Values / Range |
| --- | --- | --- |
| 0 | Analog Squelch | Low nibble, 0-9 |
| 1 | Power Save | Low nibble, 0-4 |
| 2 | VOX Level | Low nibble, 0-9 (0=Off) |
| 3 | Auto Backlight | Low nibble, 0-8 |
| 4 | TDR (Dual Watch) | Low nibble, 0=Off, 1=On |
| 5 | TOT (Tx Time Out) | Low nibble, 0-5 |
| 6 | Beep | Low nibble, 0=Off, 1=On |
| 7 | Voice Prompt | Low nibble, 0=Off, 1=On |
| 8 | Language | Low nibble, 0=English, 1=Chinese |
| 9 | Side Tone | Low nibble, 0-3 |
| 10 | Scan Mode | Low nibble, 0=Time, 1=Carrier, 2=Search |
| 11 | PTT ID (global) | Low nibble, 0-3 |
| 12 | ID Delay Time | Low nibble, 0-6 |
| 13 | Display Mode A | Low nibble, 0=Freq, 1=Channel, 2=Name |
| 14 | Display Mode B | Low nibble, 0=Freq, 1=Channel, 2=Name |
| 15 | (reserved) | 0xFF |
| 16 | Auto Lock | Low nibble, 0-3 |
| 17 | SOS Mode | Low nibble, 0=Off, 1=Alarm, 2=TX |
| 18 | Alarm Sound | Low nibble, 0=Off, 1=On |
| 19 | TDR Tx Priority | Low nibble, 0=Off, 1=Ch A, 2=Ch B |
| 20 | Tail Clear | Low nibble, 0=Off, 1=On |
| 21 | Repeater Tail Clear | Low nibble, 0-10 (value in 100ms units) |
| 22 | Repeater Detect Tail | Low nibble, 0-10 |
| 23 | Tx-Over Sound | Low nibble, 0=Off, 1=On |
| 24 | Current Work Mode | Low nibble, 0=A, 1=B |
| 25 | FM Radio | Low nibble, 0=Off, 1=On |
| 26 | Work Mode A/B | Low nibble=A (0=VFO, 1=Channel), High nibble=B |
| 27 | Key Lock | Low nibble, 0=Off, 1=On |
| 28 | Boot Screen | Low nibble, 0=Off, 1=On |
| 29 | (reserved) | 0xFF |
| 30 | R-Tone | Low nibble, 0-3 |
| 31 | Tx Start Sound | Low nibble, 0=Off, 1=On |

Part 2 (bytes 32-63):

| Offset (within part 2) | Field | Values / Range |
| --- | --- | --- |
| 0 | VOX Delay Time | Low nibble, 0-15 |
| 1 | Menu Auto Quit | Low nibble, 0-10 (0=Off, others = seconds) |
| 2 | Digital Squelch | Low nibble, 0-9 |
| 3-5 | (reserved) | 0xFF |
| 6 | STE Frequency | Low nibble, 0=55Hz, 1=259Hz |
| 7 | Weather Channel | Low nibble, 0-9 |
| 8-9 | (reserved) | 0xFF |
| 10 | Top Key Short Press | Low nibble, 0-6 (function index) |
| 11 | Side Key 2 Short Press | Low nibble, 0-7 |
| 12 | Side Key 2 Long Press | Low nibble, 0-6 |
| 13 | Side Key 3 Short Press | Low nibble, 0-6 |
| 14 | Side Key 3 Long Press | Low nibble, 0-6 |
| 15-16 | (reserved) | 0xFF |
| 17 | Keep Call Time | Bits 4:0, 0-19 (seconds) |
| 18 | (reserved) | 0xFF |
| 19 | TDR Recovery Time | Low nibble, 0-10 |
| 20-31 | (reserved) | 0xFF |

## 6.11 Basic Info Block (64 bytes, 1 packet)

The basic info block carries model identification data. Only a portion of the 64-byte block is used:

| Offset | Size | Field | Encoding |
| --- | --- | --- | --- |
| 0-7 | 8 | (reserved/padding) | 0xFF |
| 8-19 | 12 | Model Name | GB2312 encoded ASCII string, null/0xFF terminated |
| 20-27 | 8 | Model ID | ASCII decimal string, zero-padded to 8 characters |
| 28-63 | 36 | (reserved/padding) | 0xFF |

# 7. Total Data Volume Summary

For a full read or write session, the total payload bytes transferred (excluding frame overhead) are:

| Block | Packets | Bytes/Packet | Total Bytes |
| --- | --- | --- | --- |
| Handshake | 1 | 15 | 15 |
| Password | 1 | 6 | 6 |
| Version | 1 | 128 | 128 |
| DTMF | 1 | 272 | 272 |
| Encryption Keys | 1 | 264 | 264 |
| Address Book | 80 | 800 | 64,000 |
| Rx Group List | 4 | 1,024 | 4,096 |
| Channel Mode | 64 | 1,024 | 65,536 |
| VFO Mode | 1 | 128 | 128 |
| Optional Functions | 1 | 64 | 64 |
| Basic Info | 1 | 64 | 64 |
| End/Over | 1 | 2 | 2 |
| TOTAL | 157 | - | 134,575 |

At 115200 baud (approximately 11,520 bytes/second effective throughput), a full programming session transfers roughly 134 KB of payload data, which would take approximately 12-15 seconds at wire speed, plus round-trip latency for each of the 157 packet exchanges.

# 8. String / Text Encoding

Text strings in this protocol use two different encodings depending on context:

- Channel names, contact names, group names, model name: GB2312 (Guojia Biaozhun, Chinese national standard encoding). This allows both ASCII and Chinese characters to be stored. The CPS uses Encoding.GetEncoding("GB2312") for these fields.

- DTMF codes, FHSS codes, encryption keys: Custom nibble/index encoding where each byte represents a single character's position in a fixed character alphabet (e.g., "0123456789ABCD*#" for DTMF, "0123456789ABCDEF" for hex).

- Model ID: Plain ASCII decimal digits, zero-padded, 8 characters.

- Handshake string: Plain ASCII.

String termination: Strings are terminated by either a null byte (0x00) or 0xFF (unused padding). The CPS scans for either condition when reading strings back.

# 9. Radio Capacity Limits

The following limits are enforced by the CPS (derived from the Helper class constants):

| Feature | Maximum Count / Value | Notes |
| --- | --- | --- |
| Channels (total) | Determined by area x channel config | 64 packets x 16 channels = 1024 channel slots |
| Areas | Multiple (exact from ChModeHelper) | Channels divided across areas |
| Contacts (Address Book) | 4,000 (80 packets x 50/packet) | DMR IDs 1 to 16,776,415 |
| Rx Groups | 32 (4 packets x 8/packet) | Up to 32 members per group |
| Encryption Keys | 8 | Algorithms: None, Basic, Enhanced |
| DTMF Code Groups | 15 | Up to 6 digits per code |
| VFO Banks | 2 (A and B) | Each with full channel-equivalent settings |
| Frequency Range (VHF) | 18-300 MHz | Minimum 18 MHz, maximum below 300 MHz |
| Frequency Range (UHF) | 300-1000 MHz | 300 MHz to below 1000 MHz |
| DMR Color Codes | 0-15 | 16 possible values |
| DMR Time Slots | 0-1 | TS1 and TS2 |

# 10. Implementation Notes for Protocol Emulation

For anyone wishing to implement a compatible programming tool or radio emulator, the following observations are important:

## 10.1 Byte Ordering

Frequencies and most integer values use little-endian byte order within payload data. The packet framing fields (sequence number, payload length) are big-endian. CRC bytes are also big-endian (high byte first).

## 10.2 Padding Conventions

Unused bytes in every block are filled with 0xFF before data is overlaid. This means that a byte value of 0xFF reliably indicates "not programmed" or "end of data" in list structures. Tools should treat 0xFF as a sentinel and stop parsing list entries when encountered.

## 10.3 Session Must Be Sequential

The protocol does not support random access to data blocks. The radio expects the exact command sequence defined in Section 5. Skipping steps or sending commands out of order is likely to cause the radio to NAK or hang, requiring a power cycle.

## 10.4 The NAK Response

If the radio responds with command byte 0xEE in any response frame, the CPS ignores the response data and waits for the retry timer. This means the CPS will retransmit the last request up to 3 times before giving up. An emulated radio should respond with 0xEE to indicate it cannot process a request.

## 10.5 End Session Command

The End/Over command (0x01) is critical. It signals the radio to exit programming mode and return to normal operation. The payload is 2 zero bytes. Failing to send this command (e.g., if the CPS crashes) will leave the radio in programming mode until it times out internally.

## 10.6 Address Book Write Optimization

During write operations, the address book data is sent in 80 consecutive packets. The CPS does not send a batch end marker - the transition to the next phase simply happens when packet counter 79 is reached. Each packet is independently acknowledged.

# 11. Annotated Example Frames

## 11.1 Handshake Request

Complete on-wire bytes for the handshake packet (23 bytes total):

A5 02 00 00 00 0F 50 52 4F 47 52 41 4D 4A 43 38 38 31 30 44 55 [CRC_HI] [CRC_LO]

Breakdown:

- A5 = Frame header (SOF)

- 02 = Command: CmdHandshake

- 00 00 = Sequence number 0

- 00 0F = Payload length 15

- 50 52 4F 47 52 41 4D 4A 43 38 38 31 30 44 55 = "PROGRAMJC8810DU" ASCII

- [CRC_HI] [CRC_LO] = CRC-16/CCITT over bytes 1-20

## 11.2 Password Request

Complete on-wire bytes for the password packet (14 bytes total):

A5 05 00 00 00 06 FF FF FF FF FF FF [CRC_HI] [CRC_LO]

Breakdown:

- A5 = Frame header

- 05 = Command: CmdCheckPwd

- 00 00 = Sequence 0

- 00 06 = Payload length 6

- FF FF FF FF FF FF = Default blank password

- [CRC_HI] [CRC_LO] = CRC over bytes 1-11

## 11.3 Channel Write Packet (first packet)

The first channel write packet (for packet index 0) begins with:

A5 30 00 00 04 00 [1024 bytes of channel data] [CRC_HI] [CRC_LO]

Breakdown:

- A5 = Frame header

- 30 = Command: CmdWriteChMode (0x30 = 48)

- 00 00 = Sequence/packet 0 (first of 64)

- 04 00 = Payload length 1024 (0x0400 big-endian)

- [1024 bytes] = Channel data for channels 0-15

- [CRC_HI] [CRC_LO] = CRC over bytes 1 through 1029

# Appendix: File Save Format

The CPS saves and loads radio configuration to .cpf files using .NET BinaryFormatter serialization of the RadioData class. This is a proprietary binary format tied to the C# class structure and is not directly portable. The on-disk format is NOT the same as the on-wire protocol format.

The RadioData object contains instances of: BasicInfoData, ChModeData, VfoModeData, AddrBookData, RxGroupData, EncKeyData, OptionalFunData, and DtmfData. The file is a serialized object graph of these classes.

End of Report